| Discipline Name | Document Type |
| --- | --- |
| HSE | Test Paper |
| Module Name | Document Number |
| Confined Space |  |

Name:    …………………………………

Employee Number: ……………………….…

Instructor Name:  ……………….………………

Date:  ……………………………..

Marks: ………/                                        Percentage: …………  %

CONFINED SPACE — FINAL MCQ ASSESSMENT

Candidate Name: ______________________________     ID No.: ______________     Date: ____________

Instructions to candidates

1.  Answer ALL 25 questions. Each question has only ONE correct answer.

2.  Circle the letter (A, B, C or D) of the correct answer. Each correct answer scores 1 mark.

3.  Pass mark: 60 % (15 correct answers out of 25).

4.  The questions cover every section of the course notes, from 1.1 Purpose to 8.4 Electrical Equipment.

5.  Time allowed: as advised by the instructor.

- Before a confined space entry is authorised, what is the FIRST question to ask under the hierarchy of control?

A.	Which PPE should the entrants wear?

B.	How many entrants will be needed?

C.	How long will the work take?

D.	Can the work be done without entering the space?

- Which of the following can be a confined space on an oil and gas facility?

A.	A control room designed for continuous occupancy

B.	An open pipe-rack walkway with normal access

C.	A site office building

D.	A large crude oil storage tank entered through a manway

- Why is a control-room “stop” command or a DCS interlock NOT accepted as isolation of an agitator inside a vessel?

A.	It takes too long for the agitator to stop

B.	It can be defeated by a signal, a fault, an operator action or a system restart

C.	It can only be used during the day shift

D.	It must first be approved by the gas tester

- A detector with a catalytic (pellistor) LEL sensor reads 0 % LEL and 2 % oxygen inside a vessel. What does this mean?

A.	Safe — there is no flammable gas present

B.	Safe for a short entry of up to 10 minutes

C.	NOT safe — the oxygen level is deadly and the LEL reading cannot be trusted

D.	Safe after a bump test of the detector

- In a silo or hopper, what is “bridging” of stored free-flowing solids?

A.	A crust over a hidden void that can collapse when stood on

B.	A temporary walkway laid across the material

C.	A method of breaking up material from outside

D.	The ladder used to reach the silo bottom

- Which statement about gas welding and cutting (hot work) inside a confined space is correct?

A.	The pre-entry gas test alone is enough

B.	No extra permit is needed if the vessel is empty

C.	It needs a hot work permit, continuous gas monitoring and a fire watch

D.	Oxy-fuel hoses may be left inside during breaks

- A vessel has been purged with nitrogen. What must happen before anyone enters?

A.	Enter at once — nitrogen is not toxic

B.	Air-purge (ventilate) it and test the oxygen at all levels

C.	Ventilate it with more nitrogen

D.	Add pure oxygen to freshen the air

- Which pairing of role and main responsibility is correct?

A.	Site Controller – safe state of the plant; Area Authority – operates the rescue winch

B.	Site Controller – overall control of work and SIMOPS; Area Authority – safe state of the plant

C.	Site Controller – keeps the entry log; Area Authority – enters the space first

D.	Site Controller – carries out the gas test; Area Authority – acts as attendant

- Which pairing of role and main responsibility is correct?

A.	Performing Authority – applies the isolations; Area Operator – chairs the SIMOPS review

B.	Performing Authority – acts as rescue team leader; Area Operator – signs as gas tester

C.	Performing Authority – keeps the plant register; Area Operator – authorises the permit

D.	Performing Authority – controls the work party and toolbox talk; Area Operator – applies and verifies isolations

- In the hierarchy of controls used in the risk assessment, where does PPE come?

A.	It is the first and most effective control

B.	It replaces the need for isolation

C.	It is only needed when the gas test fails

D.	It is the last line of defence, used after other controls

- Which isolation method gives the highest level of protection against hazardous energy and process fluids?

A.	Positive isolation (for example, blinding or disconnection)

B.	A single closed valve

C.	A warning tag on the valve

D.	A verbal instruction to the control room

- Which statement about a confined space opening (entry and exit point) is correct?

A.	Barriers are needed only at night

B.	Ventilation ducting may be laid across the opening

C.	Barrier it as soon as the cover is removed and keep it clear of hoses and cables

D.	The opening may be left unguarded during breaks

- Why must a filter (air-purifying) respirator never be used in an oxygen-deficient atmosphere?

A.	A filter cannot create oxygen

B.	A filter is too heavy to wear inside the space

C.	A filter blocks radio communication

D.	A filter only works in hot conditions

- Contact with an entrant is lost and cannot be restored by the primary or backup method. What should the attendant do?

A.	Enter the space to check on the entrant

B.	Wait until the end of the shift

C.	Raise the alarm and activate the rescue plan — do not enter

D.	Send another worker in to check

- Why must chemicals never be mixed inside a confined space?

A.	Mixing only makes the chemicals less effective

B.	Mixing is allowed if the ventilation is running

C.	Mixed chemicals cannot be removed from site

D.	Some mixtures give off toxic gas or heat (e.g. acid with sulphide gives H2S)

- Who must be recorded in the confined space entry log?

A.	Only the supervisor

B.	Every person who enters and leaves the space

C.	Only the first person to enter

D.	Only visitors to the site

- Who can stop confined space work if they see an unsafe condition?

A.	Only the Area Authority

B.	Only the Performing Authority

C.	Anyone involved in the work

D.	Only the safety officer

- Which rescue method is preferred, and when must it be planned?

A.	Non-entry rescue using the retrieval system, planned before the permit is issued

B.	Entry rescue by the nearest colleague, planned after an emergency

C.	Entry rescue by the attendant, planned at the end of the shift

D.	Waiting for the casualty to recover, with no plan needed

- If a gas detector alarm sounds while people are inside the space, what should happen first?

A.	Investigate the cause while people stay inside

B.	Evacuate the space first, then investigate

C.	Switch off the alarm and continue work

D.	Wait five minutes to see if the alarm stops

- Apart from the set refresher interval, when should confined space refresher training also be given?

A.	Only when the person changes employer

B.	Never — initial training is valid for life

C.	Only when the person asks for it

D.	After an incident or near miss involving confined space entry

- A worker has completed confined space training but has not been formally authorised. May they act as an attendant?

A.	No — only an authorised person may perform the role

B.	Yes — the training certificate is enough

C.	Yes — if a supervisor is nearby

D.	Yes — for jobs shorter than one hour

- Which is an additional competency for personnel entering confined spaces to support integrity (inspection) work?

A.	Operating mobile cranes

B.	Recognising pyrophoric scale, NORM scale and hazardous deposits

C.	Driving heavy goods vehicles

D.	Preparing the site catering plan

- How should contractor training and competency records be checked?

A.	Accepted verbally from the contractor supervisor

B.	Checked only after an incident

C.	Verified at mobilisation, not accepted on assurance

D.	Not checked — contractors keep their own records

- What type of fall protection equipment must be used for confined space entry and rescue?

A.	A full body harness

B.	A waist safety belt

C.	A hard hat chin strap only

D.	Gloves with good grip

- What is meant by “intrinsically safe” (IS) electrical equipment?

A.	Its energy is too low to ignite a flammable atmosphere, even with a fault

B.	It has a plastic case, so it cannot give a shock

C.	It only works when connected to mains power

D.	It has been switched off and locked out

ANSWER KEY

Confined Space — Final MCQ Assessment (25 Questions)

| Question No. | Correct Answer | Course Note Ref. | Course Note Section Title |
| --- | --- | --- | --- |
| 1 | D | 1.1 | Purpose |
| 2 | D | 1.2 / 1.3 | Characteristics; Types of Confined Space |
| 3 | B | 2.1 | Physical Hazards |
| 4 | C | 2.2 / 4.3 | Atmospheric Hazards; Gas Testing |
| 5 | A | 2.3 | Engulfment |
| 6 | C | 2.4 / 5.2 | Fire & Explosion Hazards; Gas Welding & Cutting |
| 7 | B | 2.5 / 4.8 | Asphyxiation Hazard; Ventilation |
| 8 | B | 3.1 / 3.2 | Site Controller; Area Authority |
| 9 | D | 3.3 / 3.4 | Performing Authority; Area Operator |
| 10 | D | 4.1 / 4.7 | Risk Assessment; PPE |
| 11 | A | 4.2 | Safe Isolation & Reinstatement of Plant |
| 12 | C | 4.4 / 8.1 | Access & Egress; Entry & Exit |
| 13 | A | 4.5 | Breathing Apparatus |
| 14 | C | 4.6 | Communications |
| 15 | D | 5.1 | Hazardous Substances |
| 16 | B | 5.3 | Permit Durations |
| 17 | C | 5.4 | Changing Working Conditions |
| 18 | A | 6.1 / 8.2 | Rescue Plan; Retrieval Equipment |
| 19 | B | 6.2 | Emergency Arrangements & Rescue Procedures |
| 20 | D | 7.1 | Training Requirements |
| 21 | A | 7.2 | Competency Management |
| 22 | B | 7.3 | Integrity Management Competencies |
| 23 | C | 7.4 | Training & Competency Records Management |
| 24 | A | 8.3 | Fall Protection |
| 25 | A | 8.4 | Electrical Equipment |

Answer distribution: A = 7, B = 6, C = 6, D = 6.   Pass mark: 15 / 25 (60 %).

SYLLABUS COVERAGE MATRIX

All 34 Table of Contents sub-sections (1.1 to 8.4) are covered. Nine questions each test two related sub-sections.

| TOC Ref. | Course Note Section | Question No. |
| --- | --- | --- |
| 1.0 | Overview of Confined Space | 2 questions: Q1, Q2 |
| 1.1 | Purpose | Q1 |
| 1.2 | Characteristics | Q2 |
| 1.3 | Types of Confined Space | Q2 |
| 2.0 | Confined Space Hazards | 5 questions: Q3, Q4, Q5, Q6, Q7 |
| 2.1 | Physical Hazards | Q3 |
| 2.2 | Atmospheric Hazards | Q4 |
| 2.3 | Engulfment | Q5 |
| 2.4 | Fire & Explosion Hazards | Q6 |
| 2.5 | Asphyxiation Hazard | Q7 |
| 3.0 | Roles & Responsibilities | 2 questions: Q8, Q9 |
| 3.1 | Site Controller | Q8 |
| 3.2 | Area Authority | Q8 |
| 3.3 | Performing Authority | Q9 |
| 3.4 | Area Operator | Q9 |
| 4.0 | Permitted Confined Space Entry Preparation | 7 questions: Q4, Q7, Q10, Q11, Q12, Q13, Q14 |
| 4.1 | Risk Assessment | Q10 |
| 4.2 | Safe Isolation & Reinstatement of Plant | Q11 |
| 4.3 | Gas Testing | Q4 |
| 4.4 | Access & Egress | Q12 |
| 4.5 | Breathing Apparatus | Q13 |
| 4.6 | Communications | Q14 |
| 4.7 | PPE | Q10 |
| 4.8 | Ventilation | Q7 |
| 5.0 | Safe Working Practices during CSE | 4 questions: Q6, Q15, Q16, Q17 |
| 5.1 | Hazardous Substances | Q15 |
| 5.2 | Gas Welding & Cutting | Q6 |
| 5.3 | Permit Durations | Q16 |
| 5.4 | Changing Working Conditions | Q17 |
| 6.0 | Emergency & Rescue Arrangements | 2 questions: Q18, Q19 |
| 6.1 | Rescue Plan | Q18 |
| 6.2 | Emergency Arrangements & Rescue Procedures | Q19 |
| 7.0 | Training & Competency | 4 questions: Q20, Q21, Q22, Q23 |
| 7.1 | Training Requirements | Q20 |
| 7.2 | Competency Management | Q21 |
| 7.3 | Integrity Management Competencies | Q22 |
| 7.4 | Training & Competency Records Management | Q23 |
| 8.0 | Auxiliary Equipment | 4 questions: Q12, Q18, Q24, Q25 |
| 8.1 | Entry & Exit | Q12 |
| 8.2 | Retrieval Equipment | Q18 |
| 8.3 | Fall Protection | Q24 |
| 8.4 | Electrical Equipment | Q25 |

For instructor use only — remove the Answer Key and Coverage Matrix pages before issuing the paper to candidates.