| Discipline Name | Document Type |
| --- | --- |
| HSE | Test Paper |
| Module Name | Document Number |
| Confined Space |  |

Name:    …………………………………

Employee Number: ……………………….…

Instructor Name:  ……………….………………

Date:  ……………………………..

Marks: ………/                                        Percentage: …………  %

CONFINED SPACE — FINAL MCQ ASSESSMENT

Candidate Name: ______________________________     ID No.: ______________     Date: ____________

Instructions to candidates

1.  Answer ALL 60 questions. Each question has only ONE correct answer.

2.  Circle the letter (A, B, C or D) of the correct answer. Each correct answer scores 1 mark.

3.  Pass mark: 60 % (36 correct answers out of 60).

4.  The questions cover every section of the course notes, from 1.1 Purpose to 8.4 Electrical Equipment.

- Before a confined space entry is authorised, what is the FIRST question to ask under the hierarchy of control?

A.	Can the work be done without entering the space?

B.	Which PPE should the entrants wear?

C.	How many entrants will be needed?

D.	How long will the work take?

- What is the most common single cause of multiple fatalities in confined spaces?

A.	A flat battery in the gas detector

B.	Poor lighting inside the space

C.	An untrained colleague going in to attempt a rescue

D.	Slips and trips at the manway

- According to OSHA, which of the following is one of the conditions that makes a space a confined space?

A.	It is always located below ground level

B.	It is larger than 10 cubic metres

C.	It is painted with a yellow warning colour

D.	It has limited or restricted means of entry or exit

- Which of the following is NOT used to decide whether a space is a confined space?

A.	Restricted means of entry or exit

B.	Not designed for continuous occupancy

C.	The physical size of the space

D.	Large enough for a person to enter and do work

- Which of the following is an example of a BELOW-GROUND confined space?

A.	A separator on a skid

B.	A distillation column

C.	A heat exchanger shell

D.	A valve pit or meter chamber

- A habitat enclosure built around a work area for hot work is an example of:

A.	A restricted space that never needs a permit

B.	A space that becomes a confined space because of the work

C.	A workplace designed for continuous occupancy

D.	An open-air area with no atmospheric risk

- Why is a control-room “stop” command or a DCS interlock NOT accepted as isolation of an agitator inside a vessel?

A.	It takes too long for the agitator to stop

B.	It can only be used during the day shift

C.	It must first be approved by the gas tester

D.	It can be defeated by a signal, a fault, an operator action or a system restart

- During entry into a hot vessel, what is usually the FIRST sign of heat stress that the attendant notices?

A.	A change in the entrant’s voice or a slower reply on the radio

B.	The entrant’s gas detector goes into alarm

C.	The entrant asks for more tools

D.	The ventilation fan becomes noisier

- What is the normal oxygen concentration in fresh air?

A.	16.0 %

B.	20.9 %

C.	19.5 %

D.	23.5 %

- An atmosphere is considered oxygen-ENRICHED when oxygen is above:

A.	20.9 %

B.	19.5 %

C.	23.5 %

D.	25.0 %

- Why is hydrogen sulphide (H2S) especially dangerous to a person relying on their sense of smell?

A.	It smells pleasant, so people stay longer

B.	At higher concentrations it paralyses the sense of smell

C.	It is always visible as a yellow cloud

D.	It is lighter than air and disappears quickly

- Hydrogen sulphide (H2S) is heavier than air. Where is it most likely to collect in a confined space?

A.	Near the roof of the space

B.	At the bottom of the space

C.	Only near the entry manway

D.	Evenly spread at all levels

- A gas detector shows 0 % LEL and 2 % oxygen. What does this mean?

A.	The space is safe because there is no flammable gas

B.	The space is safe for a short entry only

C.	The space is NOT safe — the oxygen level is deadly

D.	The detector only needs a bump test

- In a silo or hopper, what is “bridging” of stored free-flowing solids?

A.	A temporary walkway laid across the material

B.	A method of breaking up material from outside

C.	The ladder used to reach the silo bottom

D.	A crust over a hidden void that can collapse when stood on

- Inside a confined space, the fire and explosion control strategy mainly concentrates on:

A.	Removing all oxygen while people are working

B.	Removing the fuel and excluding ignition sources

C.	Keeping fire extinguishers inside the space only

D.	Working faster to reduce the time inside

- How should pyrophoric iron sulphide deposits be handled when a vessel is opened?

A.	Let them dry in the air before removal

B.	Sweep them up dry with a steel brush

C.	Leave them in place, as they are harmless

D.	Keep them wet and remove them under control

- Which of the following is a hazard CREATED by the work itself inside a confined space?

A.	Welding fumes and solvent vapour

B.	Residual product left in the vessel

C.	Natural ground water

D.	The shape of the vessel

- A system has been purged with nitrogen. How should the atmosphere inside be treated?

A.	As safe, because nitrogen is not toxic

B.	As potentially lethal until it has been tested

C.	As safe after one hour of waiting

D.	As safe if no smell is noticed

- Which role normally chairs the SIMOPS review and confirms that rescue resources are available and not committed to another job?

A.	Entrant

B.	Attendant

C.	Site Controller

D.	Area Operator

- Which role is responsible for the state of the plant (process, isolations and permit control in the area)?

A.	Entrant

B.	Attendant

C.	Area Authority

D.	Gas tester

- Who accepts the permit, conducts the toolbox talk and supervises the work party during the entry?

A.	Site Controller

B.	Performing Authority

C.	Gas tester

D.	Emergency Response Team

- If an entrant collapses inside the space, what must the attendant NOT do?

A.	Enter the space to rescue the entrant

B.	Raise the alarm

C.	Call the rescue team

D.	Keep watching the space and the other entrants

- Which role physically applies the isolations, opens and checks the bleeds, and reports the completed plant preparation to the Area Authority?

A.	Area Operator

B.	Entrant

C.	HSE Advisor

D.	Rescue team leader

- In the hierarchy of controls, which control is the LAST line of defence?

A.	Elimination

B.	PPE and RPE

C.	Engineering controls such as isolation

D.	Substitution

- How should a Job Safety Analysis (JSA) for a confined space job be prepared?

A.	By the office alone, then sent to the crew

B.	With the people who will actually do the work

C.	Only after the work has been completed

D.	By the attendant alone, at the manway

- Which isolation method gives the highest level of protection against hazardous energy and process fluids?

A.	A single closed valve

B.	A warning tag on the valve

C.	Positive isolation (for example, blinding or disconnection)

D.	A verbal instruction to the control room

- Which statement about Lock-Out / Tag-Out (LOTO) is correct?

A.	A tag alone is enough to secure an isolation

B.	A supervisor may remove any lock to save time

C.	Locks are needed for electrical equipment only

D.	Never remove another person’s lock

- What is the correct sequence for gas testing a confined space atmosphere?

A.	Toxic gas, then oxygen, then flammable gas

B.	Oxygen, then flammable gas, then toxic gas

C.	Flammable gas, then toxic gas, then oxygen

D.	Any order, as long as all three are tested

- Why must oxygen be tested before flammable gas when using a catalytic (pellistor) sensor?

A.	The pellistor sensor measures toxic gas only

B.	The oxygen sensor takes the longest to warm up

C.	Oxygen readings are needed for the entry log only

D.	The pellistor sensor needs oxygen to give a correct reading

- What is the purpose of a bump test on a gas detector?

A.	To confirm the sensors and alarms respond to gas

B.	To adjust the sensor readings to a known gas value

C.	To charge the detector battery

D.	To record the readings in the entry log

- When recording gas test results on the permit, what should be written?

A.	The actual number shown on the detector

B.	Only the word ‘OK’

C.	A tick in the box

D.	The reading from the previous day

- Why is a ladder alone NOT acceptable as the rescue route from a vertical-entry space?

A.	Ladders are too expensive to use for rescue

B.	An unconscious casualty cannot climb, so a retrieval system is needed

C.	Ladders are not allowed inside any vessel

D.	The attendant needs the ladder to go in and help

- Why must a filter (air-purifying) respirator never be used in an oxygen-deficient atmosphere?

A.	A filter is too heavy to wear inside the space

B.	A filter blocks radio communication

C.	A filter cannot create oxygen

D.	A filter only works in hot conditions

- When the low-pressure warning sounds on a self-contained breathing apparatus (SCBA), what does it mean?

A.	The wearer has a full cylinder left to work

B.	The set needs a bump test before use

C.	The wearer should already be out of the space

D.	The wearer should start the next task

- Which of the following should NOT be used as the primary means of communication with entrants?

A.	Two-way radio

B.	A mobile phone

C.	Voice and visual contact

D.	Rope (line) signals

- Contact with an entrant is lost and cannot be restored by the primary or backup method. What should the attendant do?

A.	Raise the alarm and activate the rescue plan — do not enter

B.	Enter the space to check on the entrant

C.	Wait until the end of the shift

D.	Send another worker in to check

- How should chemical-protective gloves be selected for confined space work?

A.	Against the specific chemical, using permeation and breakthrough data

B.	Any thick glove protects against all chemicals

C.	By colour, to match the coveralls

D.	The cheapest gloves available on site

- What must entrants do if mechanical ventilation fails during the work?

A.	Continue working and report it at the end of the job

B.	Open more manways and keep working

C.	Wait inside until the fan is repaired

D.	Evacuate the space immediately

- Why must pure oxygen NEVER be used to ventilate a confined space?

A.	It is too expensive

B.	It creates an oxygen-enriched atmosphere and a serious fire risk

C.	It makes the air too cold to breathe

D.	It cannot be measured by a gas detector

- What is a “dead zone” in a ventilated confined space?

A.	The barriered area outside the opening

B.	The area directly in front of the fan

C.	An area the airflow does not reach, so contaminants remain

D.	The muster point used in an evacuation

- Why must chemicals never be mixed inside a confined space?

A.	Mixing only makes the chemicals less effective

B.	Mixing is allowed if the ventilation is running

C.	Some mixtures give off toxic gas or heat (e.g. acid with sulphide gives H2S)

D.	Mixed chemicals cannot be removed from site

- What must be done with gas welding and cutting equipment at every break?

A.	Remove it from the space and close the cylinder valves

B.	Leave it inside, ready for the next task

C.	Leave the torch lit on a low flame

D.	Close the torch valves and leave the hoses inside

- Which statement about hot work inside a confined space is correct?

A.	The pre-entry gas test alone is enough

B.	No extra permit is needed if the vessel is empty

C.	It needs a hot work permit, continuous gas monitoring and a fire watch

D.	A fire watch is only needed after working hours

- Who must be recorded in the confined space entry log?

A.	Only the supervisor

B.	Only the first person to enter

C.	Only visitors to the site

D.	Every person who enters and leaves the space

- A confined space entry permit is valid:

A.	Until its expiry time, whatever happens

B.	For a full week once it has been signed

C.	Until the job is finished, even if conditions change

D.	Only while the conditions stated on it continue to exist

- Who can stop confined space work if they see an unsafe condition?

A.	Anyone involved in the work

B.	Only the Area Authority

C.	Only the Performing Authority

D.	Only the safety officer

- The oxygen reading is falling slowly, but no alarm has sounded yet. What should the entrants do?

A.	Leave the space and find out why

B.	Keep working until the alarm sounds

C.	Move to another part of the space

D.	Switch off the detector to stop false alarms

- When must the rescue plan for a confined space entry be prepared?

A.	After the first entrant goes in

B.	Only after an emergency happens

C.	Before the permit is issued

D.	At the end of the shift

- Which type of rescue is always preferred for a confined space casualty?

A.	Entry rescue by the nearest colleague

B.	Entry rescue by the attendant

C.	Non-entry rescue using the retrieval system

D.	Waiting for the casualty to recover

- If a gas detector alarm sounds while people are inside the space, what should happen first?

A.	Investigate the cause while people stay inside

B.	Switch off the alarm and continue work

C.	Wait five minutes to see if the alarm stops

D.	Evacuate the space first, then investigate

- During non-entry rescue, the attendant feels strong resistance while winching the casualty. What should the attendant do?

A.	Stop and avoid excessive force — the casualty may be snagged

B.	Apply more force and pull harder

C.	Cut the retrieval line

D.	Enter the space to free the casualty

- Apart from the set refresher interval, when should confined space refresher training also be given?

A.	Only when the person changes employer

B.	After an incident or near miss involving confined space entry

C.	Never — initial training is valid for life

D.	Only when the person asks for it

- A worker has completed confined space training but has not been formally authorised. May they act as an attendant?

A.	Yes — the training certificate is enough

B.	Yes — if a supervisor is nearby

C.	Yes — for jobs shorter than one hour

D.	No — only an authorised person may perform the role

- Which is an additional competency for personnel entering confined spaces to support integrity (inspection) work?

A.	Recognising pyrophoric scale, NORM scale and hazardous deposits

B.	Operating mobile cranes

C.	Driving heavy goods vehicles

D.	Preparing the site catering plan

- How should contractor training and competency records be checked?

A.	Accepted verbally from the contractor supervisor

B.	Checked only after an incident

C.	Not checked — contractors keep their own records

D.	Verified at mobilisation, not accepted on assurance

- When must hard barriers and signage be placed around a confined space opening?

A.	As soon as the cover is removed

B.	Only at night

C.	Only while entrants are inside

D.	At the end of the job

- A retrieval line cannot be used because it would snag on the vessel internals. What is required?

A.	Continue with no rescue arrangement

B.	Tie the line to the vessel internals

C.	Let the attendant enter if needed

D.	Record why, and provide another rescue method (e.g. standby rescue team)

- What type of fall protection equipment must be used for confined space entry and rescue?

A.	A waist safety belt

B.	A hard hat chin strap only

C.	Gloves with good grip

D.	A full body harness

- What must be done with a harness or lanyard that has arrested a fall?

A.	Inspect it and continue using it

B.	Wash it and return it to the store

C.	Remove it from service immediately and permanently

D.	Keep it for use on short jobs only

- What is meant by “intrinsically safe” (IS) electrical equipment?

A.	It has a plastic case, so it cannot give a shock

B.	Its energy is too low to ignite a flammable atmosphere, even with a fault

C.	It only works when connected to mains power

D.	It has been switched off and locked out

ANSWER KEY

Confined Space — Final MCQ Assessment (60 Questions)

| Question No. | Correct Answer | Course Note Ref. | Course Note Section Title |
| --- | --- | --- | --- |
| 1 | A | 1.1 | Purpose |
| 2 | C | 1.1 | Purpose |
| 3 | D | 1.2 | Characteristics |
| 4 | C | 1.2 | Characteristics |
| 5 | D | 1.3 | Types of Confined Space |
| 6 | B | 1.3 | Types of Confined Space |
| 7 | D | 2.1 | Physical Hazards |
| 8 | A | 2.1 | Physical Hazards |
| 9 | B | 2.2 | Atmospheric Hazards |
| 10 | C | 2.2 | Atmospheric Hazards |
| 11 | B | 2.2 | Atmospheric Hazards |
| 12 | B | 2.2 | Atmospheric Hazards |
| 13 | C | 2.2 | Atmospheric Hazards |
| 14 | D | 2.3 | Engulfment |
| 15 | B | 2.4 | Fire & Explosion Hazards |
| 16 | D | 2.4 | Fire & Explosion Hazards |
| 17 | A | 2.5 | Asphyxiation Hazard |
| 18 | B | 2.5 | Asphyxiation Hazard |
| 19 | C | 3.1 | Site Controller |
| 20 | C | 3.2 | Area Authority |
| 21 | B | 3.3 | Performing Authority |
| 22 | A | 3.3 | Performing Authority |
| 23 | A | 3.4 | Area Operator |
| 24 | B | 4.1 | Risk Assessment |
| 25 | B | 4.1 | Risk Assessment |
| 26 | C | 4.2 | Safe Isolation & Reinstatement of Plant |
| 27 | D | 4.2 | Safe Isolation & Reinstatement of Plant |
| 28 | B | 4.3 | Gas Testing |
| 29 | D | 4.3 | Gas Testing |
| 30 | A | 4.3 | Gas Testing |
| 31 | A | 4.3 | Gas Testing |
| 32 | B | 4.4 | Access & Egress |
| 33 | C | 4.5 | Breathing Apparatus |
| 34 | C | 4.5 | Breathing Apparatus |
| 35 | B | 4.6 | Communications |
| 36 | A | 4.6 | Communications |
| 37 | A | 4.7 | PPE |
| 38 | D | 4.8 | Ventilation |
| 39 | B | 4.8 | Ventilation |
| 40 | C | 4.8 | Ventilation |
| 41 | C | 5.1 | Hazardous Substances |
| 42 | A | 5.2 | Gas Welding & Cutting |
| 43 | C | 5.2 | Gas Welding & Cutting |
| 44 | D | 5.3 | Permit Durations |
| 45 | D | 5.3 | Permit Durations |
| 46 | A | 5.4 | Changing Working Conditions |
| 47 | A | 5.4 | Changing Working Conditions |
| 48 | C | 6.1 | Rescue Plan |
| 49 | C | 6.1 | Rescue Plan |
| 50 | D | 6.2 | Emergency Arrangements & Rescue Procedures |
| 51 | A | 6.2 | Emergency Arrangements & Rescue Procedures |
| 52 | B | 7.1 | Training Requirements |
| 53 | D | 7.2 | Competency Management |
| 54 | A | 7.3 | Integrity Management Competencies |
| 55 | D | 7.4 | Training & Competency Records Management |
| 56 | A | 8.1 | Entry & Exit |
| 57 | D | 8.2 | Retrieval Equipment |
| 58 | D | 8.3 | Fall Protection |
| 59 | C | 8.3 | Fall Protection |
| 60 | B | 8.4 | Electrical Equipment |

Answer distribution: A = 15, B = 15, C = 15, D = 15.   Pass mark: 36 / 60 (60 %).

SYLLABUS COVERAGE MATRIX

Every Table of Contents item from 1.0 to 8.4 is covered by at least one question.

| TOC Ref. | Course Note Section | Question No. |
| --- | --- | --- |
| 1.0 | Overview of Confined Space | 6 questions (Q1–Q6) |
| 1.1 | Purpose | Q1, Q2 |
| 1.2 | Characteristics | Q3, Q4 |
| 1.3 | Types of Confined Space | Q5, Q6 |
| 2.0 | Confined Space Hazards | 12 questions (Q7–Q18) |
| 2.1 | Physical Hazards | Q7, Q8 |
| 2.2 | Atmospheric Hazards | Q9, Q10, Q11, Q12, Q13 |
| 2.3 | Engulfment | Q14 |
| 2.4 | Fire & Explosion Hazards | Q15, Q16 |
| 2.5 | Asphyxiation Hazard | Q17, Q18 |
| 3.0 | Roles & Responsibilities | 5 questions (Q19–Q23) |
| 3.1 | Site Controller | Q19 |
| 3.2 | Area Authority | Q20 |
| 3.3 | Performing Authority | Q21, Q22 |
| 3.4 | Area Operator | Q23 |
| 4.0 | Permitted Confined Space Entry Preparation | 17 questions (Q24–Q40) |
| 4.1 | Risk Assessment | Q24, Q25 |
| 4.2 | Safe Isolation & Reinstatement of Plant | Q26, Q27 |
| 4.3 | Gas Testing | Q28, Q29, Q30, Q31 |
| 4.4 | Access & Egress | Q32 |
| 4.5 | Breathing Apparatus | Q33, Q34 |
| 4.6 | Communications | Q35, Q36 |
| 4.7 | PPE | Q37 |
| 4.8 | Ventilation | Q38, Q39, Q40 |
| 5.0 | Safe Working Practices during CSE | 7 questions (Q41–Q47) |
| 5.1 | Hazardous Substances | Q41 |
| 5.2 | Gas Welding & Cutting | Q42, Q43 |
| 5.3 | Permit Durations | Q44, Q45 |
| 5.4 | Changing Working Conditions | Q46, Q47 |
| 6.0 | Emergency & Rescue Arrangements | 4 questions (Q48–Q51) |
| 6.1 | Rescue Plan | Q48, Q49 |
| 6.2 | Emergency Arrangements & Rescue Procedures | Q50, Q51 |
| 7.0 | Training & Competency | 4 questions (Q52–Q55) |
| 7.1 | Training Requirements | Q52 |
| 7.2 | Competency Management | Q53 |
| 7.3 | Integrity Management Competencies | Q54 |
| 7.4 | Training & Competency Records Management | Q55 |
| 8.0 | Auxiliary Equipment | 5 questions (Q56–Q60) |
| 8.1 | Entry & Exit | Q56 |
| 8.2 | Retrieval Equipment | Q57 |
| 8.3 | Fall Protection | Q58, Q59 |
| 8.4 | Electrical Equipment | Q60 |

For instructor use only — remove the Answer Key and Coverage Matrix pages before issuing the paper to candidates.